Project Report

Design and simulate a 100W Flyback converter for EV battery Charging.

Poonam Chand (231040409)

DEPARTMENT OF ELECTRICAL ENGINEERING INDIAN INSTITUTE OF TECHNOLOGY, KANPUR

August 2024 – September 2024

Design and simulate a 100W Flyback converter for EV battery Charging.

Input: 48V output 5V

Choose the appropriate transform/inductor turn ratio, magnetizing inductance, switching frequency, etc. Topology: Conventional flyback Design a single loop control with appropriate SSE, Settling time, and PM.

Show a step response in simulation and verify the setting time. From the digiky.in or mouser.in find the appropriate switching devices (Ex. MOSFET and Diode)

Fig. Circuit design for forward converter

- Bode plot of uncompensated system

  - Bode plot of compensated system

- Uncompensated circuit

- Compensated circuit

- Output voltage waveform

- Settling time

Ts= 12 msec (approx.)

- Steady state error

Ess=0.07 V i.e 1.4 %

- SWITCH SELECTION

- MOSFET SELECTION

Vds=[vg+(V0+vd)*(Np/Ns)]*2 Safety factor =2

Vds= 165 V

Irms= 5*2=10 A

For the flyback converter presented, the required minimum voltage rating of the MOSFET calculates to be 160V. An IXFH150N17T2 N-channel power MOSFET is chosen.

- Diode selection

Vd= (Vo+Vg*(Np/Ns))*2=26 V Id= 20*2=40 A

Schottky for a specific application depends mainly on the working peak reverse voltage rating, the average forward current rating of the device. An VS-42CTQ030-M3 Schottky diode is chosen

- Results

- Uncompensated system Vo=4.56 V

Wgc=3.48e10 rad/sec PM= -90 deg

GM= -149 db

unstable system

- Compensated system Vo=4.996 V

Wgc=806 rad/sec

Ts= 5 msec (theoretical) Ts= 12 sec (simulation) PM= 60.9 deg

Gm 3.94 db Stable system